Details of various GTR turbines

|  | Housing HSG type (C/T) | A/R (C/T) | Impeller C/T type TRM (C/T) C/B (T) | Thrust metal Bearing Type | Impeller Material (C/T) | 2 Price |
| --- | --- | --- | --- | --- | --- | --- |
| 14411-24U00 32 * 33 genuine ceramics | T3/T25 | 0.42/0.48 | T3/T25 50T/62t | 273° metal T25 | Aluminum Ceramics | 17.52 ten thousand |
| 14411-24U10 33 N-1 | T3/T25 | 0.42/0.64 | T3/T25 55t/62t | 360? Metal T25 | Aluminum Inconel | 22 ten thousand |
| 14411-RR595 32 * 33 In Nismo 400 R adoption | T3/T25 | 0.42/0.64 | T3/T25 55t/62t | 360° metal T25 | Aluminum Inconel | - |
| 14411-06u00 32 Nismo genuine adoption | TO 4 B/T25 | 0.42/0.64 | T3/ T 25HF 62/CD79 (15°C/B) | 273° metal T25 | Aluminum Inconel | 22 ten thousand |
| 14411-RS580 Nismo treatment varnish モ | TO 4 B/T25 | 0.42/0.64 | T3/ T 25 62/CD79 (7°C/B) | 360° metal T25 | Aluminum Inconel | 31.6 ten thousand |
| 14411-RS581 Le Mans turbine | TO 4 B/T25 | 0.60/0.64 | T3/ T 25 62/CD79 (15°C/B) | 360° metal T25 | Aluminum Inconel | 37.6 ten thousand |
| R34 standard | GT22/T25 | 0.53/0.48 | C100/T25 50/68 | Ball Bearing | Aluminum Ceramics | 17.52 ten thousand |
| R 34 N - 1 | GT22/T25 | 0.60/0.64 | C106/T25 55/62 | Ball Bearing | Aluminum Inconel | 22 ten thousand |
| GT 2510 | TO4B/T25 | 0.60/0.64 | T3/T25 63t/62t | Ball Bearing |  | 32 ten thousand |
| GT 2530 | TO4B/T25 | 0.60/0.64 | T3/T25 63t/76t | Ball Bearing |  | 33.8 ten thousand |
| GT 2540 | TO4B/T25 | 0.60/0.64 | TO 4 E/T25 46t/76t | Ball Bearing |  | 35.8 ten thousand |

The data which is published to each magazine is turned to the origin but 
Also the criss cross has come out at or the part which is proofreading mistake. 
(Specification of thrust metal of cutback quantity and 33 N1 turbines) 
In addition you verify with the other data schedule

32 * 33 * 34 genuine turbine comparisons

| Alias | R34 standard | R34-N1 | R33 standard | R33N-1 | R32 standard | R32 NISMO |
| --- | --- | --- | --- | --- | --- | --- |
| Type | C100-GT25 | C106-GT25 | T03/T25 | T03/T25 | T03/T25 | T04B/T25 |
| A / r | 0.53/0.48 | 0.60/0.64 | 0.42/0.48 | 0.42/0.64 | 0.42/0.48 | 0.42/0.64 |
| Largest adding giving | 0.93 | 0.93 | 0.84 | 0.84 | 0.75 | 0.75 |
| Turbine material | Ceramics | Metal | Ceramics | Metal | Ceramics | Metal |
| Bearing | Ball bearing | Ball bearing | Metal | Metal | Metal | Metal |

33 * 34 genuine turbine comparisons

|  |  | R34 standard | R34N - 1 | R33 standard | R33 N - 1 |
| --- | --- | --- | --- | --- | --- |
| COMPR | HSG | GT22 | GT22 | T3 | T3 |
|  | A / r | 0.53 | 0.6 | 0.42 | 0.42 |
|  | Compressor foil | C100 aluminum | C106 aluminum | T3 aluminum | T3 aluminum |
|  | Compressor foil Entrance diameter | 39.6 | 44.50 | 42.52 | 44.57 |
|  | Compressor foil major diameter | 56 | 60 | 60.10 | 60.10 |
|  | Trim (%) | 50 | 55 | 50 | 55 |
|  | Material | Aluminum | Aluminum | Aluminum | Aluminum |
| TURBIN | HSG | T25 | T25 | T25 | T25 |
|  | A / r | 0.48 | 0.64 | 0.48 | 0.64 |
|  | Turbine rotor | GT25 ceramics | GT25 inconel | T25 ceramics | T25 inconel |
|  | Trim (%) | 68 | 62 | 68 | 62 |
|  | Turbine foil major diameter | 50.47 | 53.85 | 50.47 | 53.00 |
|  | Turbine foil exit diameter | 41.73 | 42.40 | 41.73 | 41.73 |
|  | Rotor material | Ceramics | Inconel | Ceramics | Inconel |
|  | Actuator set pressure (0. 38mmst) | 460 +- 20mm H g | <- | <- | <- |
|  | Bearing | Ball Bearing | Ball Bearing | フローテング Metal | フローテング Metal |
| Supercharging pressure | (Maximum value) | 685mm H g | 685mm H g | 620mm H g | 620mm H g |

The NISSAN * Nismo genuine turbine comparison

| 14411-RS580 | 14411-RS581 | 14411-06u00 | 14411 -RR595 | 14411 -24U10 | 14411 -24U00 |
| --- | --- | --- | --- | --- | --- |
| Being sold with the Nismo, the る Turbine of 158000 Yen | Alias Le Mans turbine Turbine of 188000 Yen | To 32 Nismo limited cars It was adopted the turbine | BNR32 BCNR33 | 33 N - 1 Turbine | 32.33 Standard Ceramics |

|  |  | 14411 -RS580 | 14411 -RR581 | 14411 -06u00 | 14411 -RR595 | 14411 24U10 | 14411 -24U00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Model Type | BNR32 | BNR32 BCNR33 | BNR32 NISMO | BNR32 BCNR33 | R33GTR N-1 | R33-STD |
| COMP HSG | TYPE | T04B | T04B | T04B | T3 | T3 | T3 |
|  | A/R | 0.42 | 0.60 | 0.42 | 0.42 | 0.42 | 0.42 |
| COMP WHEEL | TYPE | T3 | T3 | T3 | T3 | T3 | T3 |
|  | TRIM | 62 | 62 | 62 | 55 | 55 | 50 |
| CENTER HSG | TYPE | T25 | T25 | T25 | T25 | T25 | T25 |
|  | Metal | F/M | F/M | F/M | F/M | F/M | F/M |
|  | Thrust Metal | 360° | 360° | 273° | 360° | 360° | 273° |
| TURBIN HSG | TYPE | T25 | T25 | T25 | T25 | T25 | T25 |
|  | A/R | 0.64 | 0.64 | 0.64 | 0.64 | 0.64 | 0.48 |
|  | Material | Ni resist | Ni resist | Ni resist | Ni resist | Ni resist | Ni resist |
| TURBIN WHEEL | TYPE | T25 | T25 | T25 | T25 | T25 | T25 |
|  | TRIM | CD79 | CD79 | CD79 | 62 | 62 | 62 |
|  | Cutting Back | 7° | 15° | 15° | ... | ... | ... |
| Actuator | Material | INCO | INCO | INCO | INCO | INCO | Ceramics |
|  | Specification | Std | Strengthening item | Std | Strengthening item | Std |  |
|  | Set pressure (Mm H g) |  | 635/760 |  | 635/760 | 460/ |  |
| Price | 1 unit cost | \158,000 | \188,000 | \110,000 |  | \110,000 | \87,600 |

Comparison with the HKS GT turbine

|  | BNR32 BCNR33 Normal | BNR32 Nismo Specification | BCNR33 N1 specification | GT2510 | GT2530 | GT2540 |
| --- | --- | --- | --- | --- | --- | --- |
| Comp A/R | TO3 0.42 | TO4B 0.42 | TO4B 0.42 | TO4B 0.60 | TO4B 0.60 | TO4B 0.60 |
| Comp TRM | TO3 50T | TO3 62t | TO3 55t | TO3 63t | TO3 63t | TO4E 46t |
| Center HSG | 270° Metal | 270° Metal | 270° Metal | Ball Bearing | Ball Bearing | Ball Bearing |
| Turbo HSG | T25 0.48 | T25 0.64 | T25 0.64 | T25 0.64 | T25 0.64 | T25 0.64 |
| Turbo Trim | T25 62t | T25HF 79t 7°C/B | T25 62t | T25 62t | GT25 76t | GT25 76t |
| Price | 87000×2 | 110000×2 | 110000×2 | 320000 | 338000 | 358000 |

The data which is published to each magazine is turned to the origin but 
Also the criss cross has come out at or the part which is proofreading mistake. 
(Specification of thrust metal of cutback quantity and 33 N1 turbines) 
In addition the schedule which is verified with the other data